DeviceServer AcquireWaveformLecroy

Author : X.Elattaoui

Current version : 1.0

Creation Date:  September 2004

Last modification

Historical modifications

| Date | Current Revue | Description | Author | Approver |
| --- | --- | --- | --- | --- |
| 18/10/2004 | 1.0 | Initial version | X.Elattaoui | A. Buteau |

1	Goal	1

2	Hardware and Software Structure	2

2.1	Hardware architecture	2

2.2	Software architecture of the DeviceServer	2

2.3	Internal Classes description :	2

2.4	Usage principle	2

3	Known Limitations of the DeviceServer AcquireWaveformLecroy	3

3.1	Possible enhancements	3

3.2	Incompatibility with RemoteDSO application	3

4	Tango Software Interface	3

4.1	Properties	3

4.2	The attributes	3

4.3	The commands	4

5	How to use the AcquireWaveformLecroy DeviceServer	4

5.1	First use of the AcquireWaveformLecroy DeviceServer	4

# Goal

The main purpose of this DeviceServer is to transfert  waveforms acquired by Lecroy scopes in the Tango world.

Both raw and scaled data of a waveform acquired on any channel of a Lecroy scope are managed by this DeviceServer.

Acquisitions has to be parametrized on the scope itself, as this deviceServer doesn’t permit to change any acquisition nor scope parameters.

# Hardware and Software Structure

## Hardware architecture

One or several Lecroy scopes as WavePro or WaveRunner series, with an Ethernet interface.

## Software architecture of the DeviceServer

The AcquireWaveformLecroy device server uses internally  the following classes

## Internal Classes description :

- SocketLecroy class:

There is only one instance of this class for 1 DeviceServer .

This class manages the socket communication  (writes/reads data ) with  the Lecroy scope.

- Waveform class :

This class sends the commands to acquire the waveform data, extract significant fields from raw data sent by the scope.

## Usage principle

- The user gives to the device :

  - The channel name : is composed by a letter followed by a number

Ex :« C2 for » channel 2, « F1 » for Fonction F1 and so on.

- The IP address of the device

- The user gets the 2 waveforms :

  - The raw data

  - And the vertically scaled data

# Known Limitations of the DeviceServer AcquireWaveformLecroy

## Possible enhancements

- The horizontal scaled data are not available yet (but can be developped if required)

- More information describing the acquisition (ex ….)  may be extracted if required.

## Incompatibility with RemoteDSO application

The scope can communicate remotely through a TCP/IP link with only 1 client application.

Pratically this means that you cannot use simultaneously :

- The RemoteDSO application provided by Lecroy for a remote control of their scopes

- A TANGO DeviceServer

Anyway RemoteDSO is not a very convenient application and using VNC to remote access the scope is a much convenient solution.

It also means, that you cannot start 2 Tango DeviceServers communicating with the same scope (it would be a strange idea to do so, but users of controls systems are sometimes strange people)

# Tango Software Interface

## Properties

The properties contain non removable parameters of the device. The user sets this parameter when he starts the DServer, it can’t be changed during the process.

| IPaddress | The IP address of the Lecroy scope. Important note : This parameter must be set for each waveform (even if in reality there is only one scope and one IP adress for all waveforms produced by this scope). This is due to the initial design choice made of showing only waveforms and not scope itself . |
| --- | --- |
| ChannelName | The name of the channel on which the waveform data will be acquired. Ex : “C2” , “F1” … |

## The attributes

| rawWaveformData | Represents the raw data of the acquired waveform |
| --- | --- |
| verticalScaledData | The vertical scaled data calculated as follow : (VERTICAL_GAIN * rawdata) – VERTICAL_OFFSET |
| waveArray1 | The length of the data array 1 |
| waveArrayCount | The total length of the acquired data. (length of data array 1 and data array 2) |
| nominalBits | Is the ADC resolution. For simple waveform its 8 bits but for complex waveform (as FFT …) its 10 up to 12 bits |
| horizontalInterval | Sampling interval for time domain waveforms |
| horizontalOffset | It’s the “delay” |
| verticalGain | The vertical gain |
| verticalOffset | The vertical offset |
| triggerTime | The timestamp (taken on the scope) of the acquisition in format MM/DD/YEAR |

## The commands

No commands

# How to use the AcquireWaveformLecroy DeviceServer

## First use of the AcquireWaveformLecroy DeviceServer

### Declare the AcquireWaveformLecroy DeviceServer on the Tango software bus

You must declare 1 Dserver per scope :Ex: “ds_ AcquireWaveformLecroy/7100”

### Declare as many Device as channel to acquire

You must declare 1 Device per channel :

Ex: “ANS-C08/DG/SCOPE7100-F1” for the channel F1

“ANS-C08/DG/SCOPE7100-C3” for the channel 3

- Give them the following  mandatory properties :

  - IPaddress : the IP address of the scope

  - ChannelName : the name of the channel of the waveform to acquire