# MicrocodeDataViewer User’s Guide

Current version of document: 1.0

Last modification: 30/07/2026

History of modifications

| Date | Revision | Description | Author |
| --- | --- | --- | --- |
| 04/08/06 | 1.0 | Initial Version | H. d’Haussy |

Table of contents

1  Applicative goals and main functionalities of the DeviceServer	2

1.1 Introduction	2

2  Technical scheme of the application	3

2.1 Software Architecture	3

2.2 Design considerations/Implementation issues	3

2.2.1 Initialization and dynamically created attributes	3

2.2.2 Reading the microcode data	3

3  TANGO software interfaces	4

3.1 Properties	4

3.2 Attributes 	4

3.3 Commands 	4

3.4 Exceptions 	5

4  Software Setup	5

5  Limitations and problems	5

5.1 Current known limitations	5

5.2 Troubleshooting 	6

# Applicative goals and main functionalities of the DeviceServer

## Introduction

The purpose of this device is to provide a generic way to display any GalilBox microcode variables data as tango attributes.

This device is able to read/write microcode variables from/to the GalilBox.

The MicrocodeDataViewer dynamically creates the Tango attributes corresponding to the microcode variables that should be displayed, those variables being defined in the data device property.

The MicrocodeDataViewer is able to handle read only attributes and read/write attributes. The type of the dynamically created attribute is defined in the device property.

Here are a few examples of what the MicrocodeDataViewer is able to do:

| Data Property item | Created attribute (result) |
| --- | --- |
| RatioAna1:DOUBLE:READ: RatioAna[1] | Creates a Tango::DevDouble read only attribute named RatioAna1. Getting this attribute’s value will read the microcode variable RatioAna[1] . |
| AcquTpp:DOUBLE:READ_WRITE:AcquTpp | Creates a Tango::DevDouble read/write attribute named AcquTpp. Getting this attribute’s value will read the microcode variable AcquTpp. Setting this attribute’s value will write the microcode variable AcquTpp. |
| MrsvMode:LONG:READ_WRITE: MrsvMode | Creates a Tango::DevLong read/write attribute named MrsvMode that will read/write the microcode variable MrsvMode. |
| Stat0Pos:BOOLEAN:READ: Stat[0],0x10000 | Creates a Tango::DevBoolean read only attribute named Stat0Pos which value will be the bit 16 of the microcode value Stat[0]. (0x10000 = 2^16) |

Note 1: The line feeds in items examples are for presentation only. Do not put carriage return within an item in the data property as carriage return is the separator between items.

Note 2: All the keywords and names are case-sensitive

# Technical scheme of the application

## Software Architecture

MicrocodeAttr represents a dynamic attributes inherited from the Tango::Attr class.

Each class reprensting a typed attribute (MicrocodeAttrDouble, MicrocodeAttributeLong and MicrocodeAttrBoolean) are inherited from the MicrocodeAttr class. These classes implement the read() and write() Tango::Attr member functions to read and write the microcode variables on the GalilBox.

## Design considerations/Implementation issues

### Initialization and dynamically created attributes

The dynamic attributes are generated during the device initialization. One attribute is created for each line of the Data property. The dynamic attributes are destroyed in the device’s destructor.

### Reading the microcode data

The microcode variables are read/written using the “WriteRead” command of the GalilBox device to send the commands <var name>=? for reading and <var name>=<value> for writing.

All the values exchanged with the microcode are ASCII floating point.

The values are read/written on the microcode each time the corresponding Tango attribute is read/written (synchronous reading/writing).

The Boolean values are written using following algorithm:

- Read the microcode variable

- Update the corresponding bit

- Write the microcode variable

Note : The way the Boolean values are written could cause a problem if the same variable is updated by the microcode or another process between step 1 and step 2. This would lose the other’s process update.

# TANGO software interfaces

## Properties

POGO generated documentation

| GalilBoxDeviceName | The complete name of the GalilBox device Example: Test/GalilBox/dev1 |
| --- | --- |
| Data | Describe the data that should be read/write on the GalilBox. For each line of the property the device will create an attribute with the specified name and type. Line format : <AttrName>:<DataType>:<RW >: <MicocodeVar> <AttrName> : the name of the Tango attribute that will be created. <DataType> = {BOOLEAN¦LONG¦DOUBLE} : the attribute type. <RW> = {READ¦READ_WRITE} : the read/write access. <MicrocodeVar> = <VarName>[,<VarMask>] : the name of the microcode variable. For Boolean variables the bit mask <VarMask> must be specified separated by comma (for example “MyVar,0x1000” will define a Boolean attribute linked to the bit 12 of the microcode variable “MyVar”). The bit mask can be specified in decimal or hexadecimal format. |
| MicrocodeName | The GalliBox microcode name (used for Start() command) |
| ProcessId | The GalliBox microcode process id (used for Start() and Abort() commands and for device state) |

## Attributes

No static attributes where defined. All attributes are dynamically created.

## Commands

POGO generated documentation

| Init | Initializes the device. Check the “Design considerations/Implementation issues” section for details. |
| --- | --- |
| State | Returns the device state. Possible states are RUNNING, STANDBY and FAULT. RUNNING : the microcode is running STANDBY : the microcode is not RUNNING FAULT : could not get microcode status (communication broken with the GalilBox) |
| Status | Returns the device status. A description of the device state. |
| Start | Starts the microcode (configured by MicrocodeName and ProcessId properties). |
| Abort (expert only) | Stops the microcode. |

## Exceptions

Exceptions are thrown when

- A communication error occurred during a read / write of an attribute or during a microcode command (Start, Abort).

# Software Setup

- A GalilBox server must be running and declared in the database

- Create a DeviceServer for the MicrocodeDataViewer (class: MicrocodeDataViewer) using jive. Create and assign value to necessary property (GalilBoxDeviceName). Usually you will also create a “Data” property and fill it using the right format. If the device must control the microcode create and fill the “MicrocodeName” and “ProcessId” properties.

- Launch the ds_MicrocodeDataViewer executable.

# Limitations and problems

## Current known limitations

- CREATE ONLY ONE DEVICE PER DEVICE SERVER !!! The implementation of the dynamic attributes conflicts when multiple devices are running in the same device server instance. If you need multiple devices you have to run one server for each device.

- The way the Boolean values are written could cause a problem if the same variable is updated by the microcode (or another process) and the MicrocodeDataViewer device at the same time.

- The values are read/written on the microcode each time the corresponding Tango attribute is read/written (synchronous reading/writing). For some application an asynchronous way of reading/writing the variables (using a thread) could be preferable.

## Troubleshooting