DRAFT

| Diffusion: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Version | Writer | Modifications |  | Reviewer |
| 08/04/2016 | 1.0 | J.GOUNO | initial version |  | A.BUTEAU S.MINOLLI |
| 12/04/2016 | 1.1 | J.GOUNO | last remarks |  | A.BUTEAU S.MINOLLI |
| 27/06/2016 | 1.2 | J.GOUNO | Completion of the “Operating mode” |  | A.BUTEAU S.MINOLLI |
| 30/06/2016 | 1.3 | J.GOUNO | Device environment updated ( Figure 1) |  | A.BUTEAU S.MINOLLI |

XPSTrajectory

User Manual

Table des matières

1	Introduction	4

2	Reference Documents	5

3	Configuration	6

3.1	Reminds	6

3.1.1	XPSTrajectory Interface	6

3.1.2	Device description	6

3.1.3	Device properties	6

3.1.4	Scalar attributes	6

3.1.5	Image attributes	6

3.1.6	Device commands	7

3.1.7	Device states	8

3.2	XPS Trajectory Device Concept	9

3.3	Device Configuration	10

3.3.1	Trajectory computation algorithms	10

3.3.2	Input arguments	11

3.4	Operating mode	15

3.4.1	Using the SIXS python algorithm	15

3.4.2	Using native C++ algorithm	19

3.4.3	GetOrderedAxisList command	34

3.4.4	Reset command	34

3.4.5	Stop command	34

Illustrations tables

Figure 1 : Device environment	9

Figure 2 : XPSTrajectory device property	10

Figure 3 :XPSTrajectory-  Init command	10

Figure 4 : Pt image attribute	12

Figure 5 : Pvt image attribute	13

Figure 6 : rawPVT image attribute	14

Figure 7 : Upload data into XPS Controller	15

Figure 8 : Check Validity	16

Figure 9 : Trajectory check OK	16

Figure 10 : Trajectory check Error	17

Figure 11 : Go to origin	17

Figure 12 : uploaded raw pvt	18

Figure 13 : Start the trajectory	18

Figure 14 : Upload data into XPS Controller	19

Figure 15 : rawPVT image attribute	20

Figure 16 : Trajectory check OK	20

Figure 17 : Trajectory check Error	21

Figure 18 : Go to origin	21

Figure 19 : uploaded raw pvt	22

Figure 20 : Start the trajectory	23

Figure 21 : Upload data into XPS Controller	24

Figure 22 : Check Validity	25

Figure 23 : Trajectory check OK	26

Figure 24 : Trajectory check Error	26

Figure 25 : Go to origin	27

Figure 26 : uploaded raw pvt	27

Figure 27 : Start the trajectory	28

Figure 28 : Upload data into XPS Controller	29

Figure 29 : Check Validity	30

Figure 30 : Trajectory check OK	31

Figure 31 : Trajectory check Error	31

Figure 32 : Go to origin	32

Figure 33 : uploaded raw pvt	32

Figure 34 : Start the trajectory	33

Figure 35 : Get ordered axis list	34

Figure 36 : reset calculation	34

# Introduction

The purpose of this document is to provide additional information about using the XPSTrajectory Device.

The basic documentation of the device is in the form of html files so that the user can access the html documentation.

# Reference Documents

| Code | Reference | Title |
| --- | --- | --- |
|  | XpsTrajectory_ind1.doc | Calcul de trajectoires pour scans XPS |

Table 1 : Reference documents

# Configuration

## Reminds

### XPSTrajectory Interface

Here is the description of the XPS Trajectory Tango interface from the html documentation: properties, attributes, commands and states.

### Device description

At first run of the XPSTrajectory device, the property is generated.

### Device properties

| Property name | Property type | Description | Default value |
| --- | --- | --- | --- |
| TrajectoryAlgoType | DEV_SHORT | Enumerated type Trajectory Algorithm: (0 = native algorithm , 1 = SIXS python algorithm). | 0 |

### Scalar attributes

| Attribute name | Data Type | R/W Type | Expert |
| --- | --- | --- | --- |
| currentTrajectoryOk: TRUE, indicates that the current trajectory has been checked and can be loaded into the XPS Controller. | DEV_BOOLEAN | READ | No |

### Image attributes

| Attribute name | Data Type | X Data Length | Y Data Length | Expert |
| --- | --- | --- | --- | --- |
| pvt: PVT trajectory input. For each point you must issue all motor positions for the group in the group order. All positions must be provided absolute. You must provide only the useful points, the device computes the starting and stopping points. Time in seconds in absolute way (from the start point), position in (units), velocity in units/sec. line 0 : time 0=0, abs pos.1, speed pos.1, abs pos.2 , speed pos.2, abs pos.n , speed pos.n. line 1 : time 1, abs pos.1, speed pos.1, abs pos.2 , speed pos.2, abs pos.n , speed pos.n. line m : time m, abs pos.1, speed pos.1, abs pos.2 , speed pos.2, abs pos.n , speed pos.n. last line : time z, abs pos.1, speed pos.1, abs pos.2 , speed pos.2, abs pos.n , speed pos.n | DEV_DOUBLE | 17 | 1000 | Yes |
| pt: PT trajectory input. The device computes the velocities. For each point you must issue all motor positions for the group in the group order. All positions must be provided absolute. You must provide only the useful points, the device computes the starting and stopping points. Time in seconds in absolute way (from the start point), position in (units) line 0 : time 0=0, abs pos.1, abs pos.2 , abs pos.n line 1 : time 1, abs pos.1, abs pos.2, abs pos.n line m : time m, abs pos.1, abs pos.2, abs pos.n last line : time z, abs pos.1, abs pos.2, abs pos.n | DEV_DOUBLE | 17 | 1000 | Yes |
| rawPVT: raw PVT Trajectory in the form : T1, P1m1,V1m1, P1m2, V1m2... where Tx is an relative time Pxmi is the relative move of motor i Vxmi is the speed for motor i | DEV_DOUBLE | 17 | 1000 | Yes |
| uploadedRawPVT: uploaded raw PVT Trajectory in the form : T1, P1m1,V1m1, P1m2, V1m2... where Tx is an relative time Pxmi is the relative move of motor i. Vxmi is the speed for motor i | DEV_DOUBLE | 17 | 1000 | Yes |

### Device commands

| Command name | Description | Level | Argument In | Argument Out |
| --- | --- | --- | --- | --- |
| Init | This commands re-initialise a device keeping the same network connection. After an Init command executed on a device, it is not necessary for client to re-connect to the device. This command first calls the device delete_device() method and then execute its init_device() method. For C++ device server, all the memory allocated in the init_device() method must be freed in the delete_device() method. The language device desctructor automatically calls the delete_device() method. | Operator | DEV_VOID | DEV_VOID |
| State | This command gets the device state (stored in its device_state data member) and returns it to the caller. | Operator | DEV_VOID | DEV_STATE |
| Status | This command gets the device status (stored in its device_status data member) and returns it to the caller. | Operator | DEV_VOID | CONST_DEV_STRING |
| GotoOrigin | go to origin point of the trajectory. | Operator | DEV_VOID | DEV_VOID |
| Reset | reset the trajectory. | Operator | DEV_VOID | DEV_VOID |
| CheckValidity | Checks if the trajectory is possible « You must be at trajectory origin position” = True if trajectory is possible | Operator | DEV_VOID | DEV_BOOLEAN |
| Start | Starts the trajectory “ you must be at origin point” trajectory check must have been successful | Operator | DEV_VOID | DEV_VOID |
| Upload | Uploads the previously written "trajectory" attribute in the controller | Operator | DEV_VOID | DEV_VOID |
| Stop | Stops the execution of the running trajectory | Operator | DEV_VOID | DEV_VOID |
| GetOrderedAxisList | Gives the list of axis in the order required when setting trajectory arrays | Expert | DEV_VOID | DEVVAR_STRINGARRAY |

### Device states

FAULT: The trajectory is checked “invalid” or is not at start point.

ON: Device check trajectory is Ok

OFF: Trajectory is not initialized or not loaded.

RUNNING: trajectory is loading.

MOVING: Trajectory is going to start point or computation is in progress.

STANDBY: Trajectory has been done or waiting for startup at the start point.

## XPS Trajectory Device Concept

The XPSTrajectory device provides functions to access XPS controller for managing motion sequences.

The input data supplied by client (i.e. a User, a Tango device or any Tango compatible software device) can be arrays of several types:

- Position-time (PT),

- Position-velocity-time (PVT),

- rawPVT (pre-calculated trajectories).

The client uploads data to the device that initiates the calculations, formatting and checking. These data are sent to the XPS Controller which checks their validity. If data are valid the client is able to start the trajectory.

Figure 1 : Device environment

A PT type trajectory can be computed by using two different algorithms:

- A native algorithm, which was developed in C++, for the very first version of XPS Trajectory,

- A SIXS algorithm developed in Python and whose principle is implemented in this new version of XPS Trajectory.

## Device Configuration

### Trajectory computation algorithms

The principles used in these algorithms are different (see ref [A1] for more details):

| Native algorithm (=0) | SIXS python algorithm (=1) |
| --- | --- |
| Calculation of time at starting point | Calculation of the duration of retreat |
| Calculation of time to reach the last point | Calculation of the breaking period |
| Acceleration = 0.25 | Acceleration = 0.5 |

- The user must define in “TrajectoryAlgoType” property of the device, the type of algorithm he wishes to use (0 = native algoritm, = 1 sixs python algorithm) to calculate the trajectory. Fix the choice of the algorithm into property:

Figure 2 : XPSTrajectory device property

- The user should initialize the Device to take the change into account (by using the Init command).

Figure 3 :XPSTrajectory-  Init command

### Input arguments

The XPS Trajectory Device allows 2 types of trajectories:

- Position-velocity-time (PVT),

- Position-time (PT).

Nota:

If the SIXS python algorithm is setting into property (TrajectoryAlgoType=1), only the calculation of Pt trajectories is done using this algorithm, for other trajectory type, as Pvt, the calculation is done using the native C++ algorithm.

#### Positions and times input data

This input data includes the time and positions.
PT type trajectories are defined for each positioners (or engines axes): [<Abs Time> <Pos Abs M1> <Pos Abs M2>...<Pos Abs Mn>]:

- with : Abs Timei : absolute time at input  point i,

- Pos Abs Mi Absolute  Position at  point i (0 ≤ i<n)

From these data, the XPSTrajectory device computes relative times and relative distances and velocities at each point.

The client should select the Pt image attribute (available in expert view mode) to set and compute these data.

Figure 4 : Pt image attribute

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OUT OF RANGE:

  - acceleration = 0,

  - start delta time = 0,

  - end delta time = 0.

#### Positions, Velocity and times input data

This input data includes times, and couples position and velocity.

PVT type trajectories are defined for each positioners (or engines axes) as follows: [<Abs Time0> <Pos Abs M1> <Vel M1> <Pos Abs M2> <Vel M2> ...<Pos Abs Mn> <Vel Mn>]

- with : Abs Timei : absolute time at input  point i,

- Pos Abs Mi : Absolute  Position at  point i (0 ≤ i<n )

- Vel Mi : velocity  at  point i (0 ≤ i<n)

From these data, the XPSTrajectory device computes relative times and relatives distances at each point.

The client should select the Pvt image attribute (available in expert view mode) to set and compute these data.

Figure 5 : Pvt image attribute

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OUT OF RANGE:

  - acceleration = 0,

  - start delta time = 0,

  - end delta time = 0.

#### rawPvt

This input data includes times, and couples relative position and velocity.

This input data are pre-calculated trajectories which will be copied into the internal structures of XPSTrajectory device.

The client should select the rawPvt image attribute (available in expert view mode) to set these data.

Figure 6 : rawPVT image attribute

## Operating mode

### Using the SIXS python algorithm

The user must define in the property of the device, that he wishes to use SIXS Python algorithm, (TrajectoryAlgoType= 1) to calculate the trajectory. Fix the choice of the algorithm in property (see 3.3.1)

Remind: This algorithm is available only to compute Pt type input data.

- Step one: « Upload » into XPS Controller

Once the client has written the input data (see 3.3.2.1) it should use the Upload command to submit the computed data to XPS controller.

Figure 7 : Upload data into XPS Controller

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : XPS trajectory check condition not fulfilled [must be uploaded, stopped]

  - no valid trajectory [provide valid trajectory],

  - class not properly configured check configurations, [try to restart device].

- Step two : CheckValidity

This command is used to check data for validation after uploaded into the XPS controller.

Figure 8 : Check Validity

If the trajectory is validated by XPS Controller, the device displays the information: “Trajectory check OK”

Figure 9 : Trajectory check OK

For an invalid trajectory, the device displays the information: “Trajectory check ERROR”

Figure 10 : Trajectory check Error

- Step three : Go to origin

This command is used to initialize the origin of the trajectory.

Figure 11 : Go to origin

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - class not properly configured check configurations, try to restart device

  - error : trajectory not loaded in XPS [load it first],

  - Group not Ready to move [check Group].

- Step four: uploaded raw Pvt Trajectory

This action is optional, the client should select the uploadedRawPvt image attribute to display an overview of the computed data result.

Figure 12 : uploaded raw pvt

- Step five : Start

This command starts the trajectory motions.

Figure 13 : Start the trajectory

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : trajectory not checked  [check trajectory first],

  - class not properly configured check configurations, [try to restart device].

### Using native C++ algorithm

The user must define in the property of the device, that he wishes to use native C++ algorithm, (TrajectoryAlgoType= 0) to calculate the trajectory. Fix the choice of the algorithm in property (see 3.3.1)

#### Positions and times input data

- Step one: « Upload » into XPS Controller:

Once the client has written the input data (see 3.3.2.1) it should use the Upload command to submit the computed data to XPS controller.

Figure 14 : Upload data into XPS Controller

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specifics messages the rejected reason:

- OPERATION NOT ALLOWED:

  - error : XPS trajectory check condition not fulfilled [must be uploaded, stopped]

  - no valid trajectory [provide valid trajectory],

  - class not properly configured check configurations, [try to restart device].

- Step two : CheckValidity

This command is used to check data for validation after uploaded into the XPS controller.

Figure 15 : rawPVT image attribute

If the trajectory is validated by XPS Controller, the device displays the information: ”Trajectory check OK”

Figure 16 : Trajectory check OK

For an invalid trajectory the device displays the information: “Trajectory check ERROR”

Figure 17 : Trajectory check Error

- Step three : Go to origin

This command is used to initialize the origin of the trajectory.

Figure 18 : Go to origin

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - class not properly configured check configurations, try to restart device

  - error : trajectory not loaded in XPS [load it first],

  - Group not Ready to move [check Group].

- Step four: uploaded raw Pvt Trajectory

This action is optional, the client should select the uploadedRawPvt image attribute to display an overview of the computed data result.

Figure 19 : uploaded raw pvt

- Step five : Start

This command starts the trajectory motions.

Figure 20 : Start the trajectory

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : trajectory not checked  [check trajectory first],

  - class not properly configured check configurations, [try to restart device].

#### Position, Velocity and Times input data

- Step one: « Upload » into XPS Controller:

Once the client has written the input data (see 3.3.2.2) it should use the Upload command to submit the computed data to XPS controller.

Figure 21 : Upload data into XPS Controller

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : XPS trajectory check condition not fulfilled [must be uploaded, stopped]

  - no valid trajectory [provide valid trajectory],

  - class not properly configured check configurations, [try to restart device].

- Step two : CheckValidity

This command is used to check data for validation after uploaded into the XPS controller.

Figure 22 : Check Validity

If the trajectory is validated by XPS Controller, the device displays the information: ”Trajectory check OK”

Figure 23 : Trajectory check OK

For an invalid trajectory the device displays the information: “Trajectory check ERROR”

Figure 24 : Trajectory check Error

- Step three : Go to origin

This command is used to initialize the origin of the trajectory.

Figure 25 : Go to origin

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - class not properly configured check configurations, try to restart device

  - error : trajectory not loaded in XPS [load it first],

  - Group not Ready to move [check Group].

- Step four: uploaded raw Pvt Trajectory

This action is optional, the client should select the uploadedRawPvt image attribute to display an overview of the computed data result.

Figure 26 : uploaded raw pvt

- Step five : Start

This command starts the trajectory motions.

Figure 27 : Start the trajectory

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : trajectory not checked  [check trajectory first],

  - class not properly configured check configurations, [try to restart device].

#### rawPvt

- Step one: « Upload » into XPS Controller:

Once the client has written the input data (see 3.3.2.3) it should use the Upload command to submit the computed data to XPS controller.

Figure 28 : Upload data into XPS Controller

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : XPS trajectory check condition not fulfilled [must be uploaded, stopped]

  - no valid trajectory [provide valid trajectory],

  - class not properly configured check configurations, [try to restart device].

- Step two : CheckValidity

This command is used to check data for validation after uploaded into the XPS controller.

Figure 29 : Check Validity

If the trajectory is validated by XPS Controller, the device displays the information: ”Trajectory check OK”

Figure 30 : Trajectory check OK

For an invalid trajectory the device displays the information: “Trajectory check ERROR”

Figure 31 : Trajectory check Error

- Step three : Go to origin

This command is used to initialize the origin of the trajectory.

Figure 32 : Go to origin

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - class not properly configured check configurations, try to restart device

  - error : trajectory not loaded in XPS [load it first],

  - Group not Ready to move [check Group].

- Step four: uploaded raw Pvt Trajectory

This action is optional, the client should select the uploadedRawPvt image attribute to display an overview of the computed data result.

Figure 33 : uploaded raw pvt

- Step five : Start

This command starts the trajectory motions.

Figure 34 : Start the trajectory

Nota:

If conditions are not fulfilled to realize the trajectory, XPS trajectory device indicates by specific messages the rejection reason:

- OPERATION NOT ALLOWED:

  - error : trajectory not checked  [check trajectory first],

  - class not properly configured check configurations, [try to restart device].

### GetOrderedAxisList command

The client can use this command to get the ordered list of positioners:

Figure 35 : Get ordered axis list

### Reset command

This command resets the calculation of the trajectory.

Figure 36 : reset calculation

### Stop command

This command aborts all movements of the trajectory.